TITLE OF THE INVENTION

Low-Power Soil Moisture Sensing Node with Adaptive Sampling

CROSS-REFERENCE TO RELATED APPLICATIONS

This application claims the benefit of U.S. Provisional Application No. 63/904,118, filed October 3, 2025, which is incorporated herein by reference in its entirety.

BACKGROUND

This disclosure relates to soil moisture sensing systems for agricultural applications, and more particularly to battery-powered sensing nodes deployed in remote fields.

Conventional soil moisture sensors sample at a fixed rate and transmit every reading. Fixed-rate operation drains batteries quickly, so growers must visit each sensor several times per season to replace them.

Some systems reduce the sampling rate to save power, but a slow fixed rate misses rapid moisture changes after irrigation or rainfall. There is a need for a sensing node that samples often when conditions change and rarely when they do not.

SUMMARY

A soil moisture sensing node includes a capacitive probe, a microcontroller that samples the probe at a variable rate, and a radio that transmits periodic summaries rather than individual readings.

The microcontroller compares each reading against a threshold and increases the sampling rate when moisture changes quickly. Between sampling events the microcontroller enters a sleep mode, which allows the node to operate for a full growing season on a single battery.

BRIEF DESCRIPTION OF THE DRAWINGS

FIG. 1 is a perspective view of a soil moisture sensing node installed in soil.

FIG. 2 is a block diagram of the electronics of the node of FIG. 1.

FIG. 3 is a flow chart of the adaptive sampling method.

FIG. 4 is a timing diagram showing sampling events during an irrigation cycle.

FIG. 5 shows the format of a summary packet transmitted by the radio.

DETAILED DESCRIPTION

Referring to FIG. 1, the soil moisture sensing system 100 includes a capacitive probe 110 inserted into the soil at a depth of 10 to 30 centimeters.

The capacitive probe 110 includes a sensing element that measures the dielectric constant of the surrounding soil, which varies with moisture content.

The electronics are enclosed in a housing 104 mounted above the soil surface. The housing 104 is sealed against water ingress using a gasket 106.

As shown in FIG. 2, a microcontroller 120 samples the probe 110 at a first sampling rate, typically between 1 and 10 Hz during active monitoring.

The microcontroller 120 compares each reading to a fixed threshold value stored in memory. When a reading crosses the threshold, the microcontroller 120 doubles the sampling rate until two consecutive readings fall below it.

The threshold value can be updated based on the previous 24 readings to account for seasonal variations in baseline soil moisture.

Between sampling events, the microcontroller 120 enters a sleep mode in which the probe 110 is powered down.

A radio transmitter 210 sends a summary packet containing minimum, maximum, and average moisture readings at a second rate, typically every 15 minutes. FIG. 5 shows the packet format.

FIG. 6 shows the power consumption profile of the node over a 48-hour period, including the reduced draw during sleep mode.

With the adaptive sampling described above, the system can operate for up to 18 months on a single battery charge.

While the invention has been described with reference to specific embodiments, modifications and variations will be apparent to those skilled in the art without departing from the scope of the claims.

- WHAT IS CLAIMED IS

- A soil moisture sensing system comprising:

a capacitive probe;

a microcontroller coupled to the probe and configured to sample the probe at a variable first rate based on substantially continuous monitoring; and

a radio transmitter configured to transmit summary data at a second rate.

- The system of claim 1, wherein the capacitive probe comprises a cylindrical electrode.

- The node of claim 1, wherein the sensor housing is sealed against water ingress.

- The system of claim 1, wherein the first rate is adjusted based on detected moisture changes.

- The system of claim 1, wherein the summary data includes minimum, maximum, and average readings.

- The system of claim 1, further comprising a battery configured to power the system for at least 12 months.

- The system of claim 1, wherein the microcontroller is configured to enter a sleep mode between sampling events.

- The system of claim 1, further comprising means for transmitting the summary data to a remote server.

- The system of claim 1, wherein the probe is inserted at a depth of 10 to 30 centimeters.

- The system of claim 9, wherein the depth is 15 centimeters.

- The system of claim 9 or 10, wherein the probe comprises a temperature sensor.

- The system of claim 11, wherein the temperature sensor is calibrated.

- A method of monitoring soil moisture comprising:

inserting a capacitive probe into soil;

sampling the probe at a variable rate; and

wirelessly transmitting summary data.

- A method of sensing soil moisture, comprising:

(a) providing a soil moisture probe having a capacitive sensing element;

(b) sampling the probe with a microcontroller at a first rate;

(c) adjusting the first rate based on a machine-learned threshold; and

(d) transmitting, by a radio, a summary packet at a second rate.

ABSTRACT

A soil moisture sensing system for agricultural monitoring comprises a capacitive probe embedded in soil, a microcontroller sampling the probe at variable rates, and a radio transmitter sending periodic summaries rather than individual readings. The system adapts its sampling frequency based on detected moisture changes, sampling quickly after irrigation or rainfall and slowly when conditions are stable, to optimize power consumption. Between sampling events the microcontroller enters a sleep mode in which the probe is powered down. A threshold value stored in memory determines when the sampling rate is increased, and the threshold may be updated from recent readings to account for seasonal changes in baseline moisture. A machine learning algorithm adjusts threshold values over time. Summary packets report minimum, maximum, and average moisture at a fixed interval. The system enables long-term deployment in remote fields with minimal battery maintenance, operating for a full growing season or longer on a single charge. Applications include precision irrigation control, drought monitoring, and crop health monitoring in commercial agriculture.